# Pipelined Architecture

## 🔹 What is Pipelining? (Very Simple)

Pipelining is a technique used in CPU (processor) to do multiple tasks at the same time, just like multitasking, but in an organized way.

👉 Instead of finishing one instruction completely and then starting the next, the CPU breaks instructions into steps and works on many instructions together.

## 🏭 Real-Life Example: Factory Assembly Line

Imagine a car factory 🚗

| Worker | Job |
| --- | --- |
| Worker 1 | Install engine |
| Worker 2 | Add wheels |
| Worker 3 | Paint car |
| Worker 4 | Final check |

- One car moves step by step

- At the same time:

  - Car 1 is being painted

  - Car 2 is getting wheels

  - Car 3 is getting engine

✔️ Many cars are processed at the same time, but at different stages

👉 This is exactly how pipelining works in CPU

## 🧠 Pipelined Processor (CPU Example)

A pipeline processor divides work into stages.

Each instruction passes through these stages:

- Fetch – get instruction

- Decode – understand instruction

- Execute – do the calculation

- Memory – read/write memory

- Write Back – save result

## 📊 Basic Pipeline Diagram (Easy ASCII Diagram)

Data In

|

v

+----+   +----+   +----+   +----+   +----+

| S1 |-->| S2 |-->| S3 |-->| S4 |-->| S5 |

|Fetch|Decode|Exec |Mem |Write|

+----+   +----+   +----+   +----+   +----+

|       |        |        |        |

R1      R2       R3       R4       R5

|       |        |        |        |

Data Out

## 🔧 Diagram Components (In Simple Words)

### 1️⃣ Data In

- Instructions coming into CPU
 👉 Example: ADD A, B

### 2️⃣ Stages (S1, S2, S3…)

- Each stage does one small job

- Like workers in a factory

### 3️⃣ Registers (R1, R2...)

- Temporary storage

- Hold data between stages
 👉 Like waiting table between workers

### 4️⃣ Computation Units (C1, C2...)

- Do actual work (addition, subtraction, logic)

### 5️⃣ Control Unit

- Boss of CPU 🧠

- Tells when and what each stage should do

### 6️⃣ Data Out

- Final result
 👉 Example: Result of ADD A, B

## ⏱️ How Pipelining Works (Step by Step)

### Clock Cycle Example

| Clock | S1 | S2 | S3 |
| --- | --- | --- | --- |
| 1 | A | - | - |
| 2 | B | A | - |
| 3 | C | B | A |
| 4 | D | C | B |

👉 After pipeline is full:

- One result comes every clock cycle

- Very fast 🚀

## 🧾 Instruction Execution Example

### Instruction Steps:

- FI – Fetch Instruction

- DA – Decode Instruction

- FO – Fetch Operands

- EX – Execute

### Example:

Instruction 1 → Fetch

Instruction 2 → Fetch

Instruction 1 → Decode

Instruction 3 → Fetch

Instruction 2 → Decode

Instruction 1 → Execute

✔️ Multiple instructions run together
 ✔️ This is called Instruction Level Parallelism

## ⚠️ What are Pipeline Hazards?

Sometimes pipeline cannot work smoothly.

These problems are called Hazards.

## 🚧 1. Structural Hazard (Resource Clash)

### Real-Life Example:

- Two people want to use one bathroom 🚻 at same time

### CPU Example:

- Two instructions want to use same ALU or memory

- CPU must wait → slow down

## 🔁 2. Data Hazard (Dependency Problem)

### Real-Life Example:

- You ask result before calculation is done ❌

### CPU Example:

Instruction 1: A = B + C

Instruction 2: D = A + 5

👉 Instruction 2 needs A
 👉 But Instruction 1 is not finished yet
 ➡️ CPU must wait

### Types of Data Dependency (Easy Meaning)

| Type | Meaning |
| --- | --- |
| RAW | Read after Write (most common) |
| WAR | Write after Read |
| WAW | Write after Write |
| RAR | Read after Read (safe) |

## 🔀 3. Control Hazard (Decision Problem)

### Real-Life Example:

- You turn left/right but decision not made yet 🤔

### CPU Example:

IF (x > 0) GOTO L1

- CPU doesn’t know which instruction is next

- Wrong instruction may enter pipeline

- Pipeline may stall or flush

## 📈 Performance Factors (Simple)

### 🕒 Latency

- Time to finish one instruction

- Less latency = better

### ⚙️ Efficiency

- How busy pipeline is

- More busy = more efficient

### 🚀 Throughput

- Instructions completed per second

- Main advantage of pipelining

## ✅ Benefits of Pipelining

✔️ Faster execution
 ✔️ More instructions per second
 ✔️ Better use of CPU parts
 ✔️ Ideal for repeated tasks

## ❌ Disadvantages of Pipelining

❌ Complex design
 ❌ Branch instructions cause problems
 ❌ Hazards reduce performance
 ❌ Hard to predict exact speed

## 🧠 Final One-Line Summary

Pipelining is like a factory assembly line where many instructions are processed at the same time, each at a different stage, to make the CPU faster and more efficient.